Planification of the construction Site

- Preparation: The depth of the Excavation depends on the type of soil, the morphology of the site, but it needs to be at least 30 cm deep. The foundations need to reach the solid soil. In the center the first layer of loose earth together with all the plants (ca 10-15cm) should be replace by Hardcore filling. It is important to use appropriate Anti termite protection and Waterproofing.

- Lean concrete 10cm

- Casting form: Either made out of bent metal sheets or plywood combined with vertical elements. This depends what is available on site, one the shape of the elements and how many reproductions of one element are needed.

- Reinforcement bars: The main horizontal bars in the circular foundation have a diameter of 10mm. The linking bars have a diameter of 6mm. Furthermore, there are metal grid close to the top and the bottom of the slab. All the reinforcement bars have to be covered with at least 3cm of concrete to prevent corrugation.

- Concrete Slab: Concrete ratio 1-2-4 (one bag of cement, two of aggregate and four of sand) and has to be properly cured

- Metal pillar base U-Shape Holder: Has to be put into the concrete while still wet and linked with vertical reinforcement bars.

- Wall made of curved interlocking compressed earth bricks: every 4th joint vertically and horizontally should be reinforced with cement.

- Install the Casting form of the ring beam: could possibly made out of bent metal sheets fixed with reinforcement bars to the brick wall. But it depends on several factors described already in point 3.

- Reinforcement bars: In the same way as the circular foundation described in point 4 (main bars 10mm/linking bars 6mm)

- Ring beam 22cm x 22cm: in concrete; all the reinforcement bars have to be covered with at least 3cm of concrete to prevent corrugation.

- Metal angle: (later used for the fixation of the truss) have to be placed on top of the ring beam into the concrete while it is still wet and linked with vertical reinforcement bars. The Concrete has to be properly cured.

- Prefabricate the trusses: on the floor while the concert is curing. The trusses are made out of different elements with a maximal dimension of 2.5 x 10 x 240cm that are nailed together. The easiest way to reach a certain precision is to draw the form on the floor in order to use it as model.

- Timber pillar: are made out of four elements of 2.5 x 10 x 240cm that are screwed together. On the top the same metal angel element as in point 11. Connect the pillar to the metal u-shape holder with nails or screws.

- Scaffolding: horizontal timber planks can be used as a temporary scaffolding put on top of the circular wall. Additional vertical scaffolding might be needed depending on the height of the roof.

- Lifting the trusses up and bolt them to the metal angle element on top of the pillar and the wall. The connection shouldn’t be too tight in order to allow dilatation. One truss of the Library-Type building (0,055m3) weights between 30kg - 40kg depending on the used wood and is therefore easily liftable and can be hold in place by two people.

- Central connection: In the centre the trusses fixed with bolts to metal plates that are weld to a circular metal element. The trusses just need to be hold in place and shouldn’t be tighten too much in order to allow dilatation.

- Horizontal Battens: nailed to the trusses.

- Light colored PVC layer: Water proofing protection. Not mandatory when the thatch layer is at least 40cm thick and properly done. Taking advantage of the conical shape of the roof, concentric horizontal rings can stay in place due to the geometry. The different “PVC rings” getting always smaller towards the top and each should always overlap the next one. The PVC should be white or as light colored as possible in order to reflect light.

- A cords net can be used for the fixation of the thatch, that needs quite a small grid (one horizontal element each 30 cm). Like in point 18, the conical form can be used to its advantage.

- Thatch: has to be at least 40cm thick if the PVC layer isn’t used, otherwise 10-20cm is enough. The basic bunches have to be prepared before on the floor and then tighten to the cord net on the roof.

- Finishing: floor screed paving on concrete. The slope of at least 1.5 degrees towards the edge.
Earth plastering the inside of the wall. Chicken wire can be used to prevent cracks. Doors and other furniture done out of wood. Safety protection might need to be added.