Auxiliary material:

S1. Sensor pole design and injection procedure

To maintain intimate contact with the snow and to reduce vertical cross contamination, the carbon monoxide sensors were installed on tapered poles (radius decreasing 1 cm per meter of length) that could be manually inserted to 1m depth in a variety of snow textures. The material employed for the structural element was selected to have thermal conductivity equal to typical snow packs, be durable to impacts at low temperatures, and be light in color to minimize absorption of solar radiation. A 50 mm o.d., 25 mm i.d. tube of polyamide plastic (PA6) satisfied these requirements. The seven sensors were mounted flush with the surface of the pipe at 15 cm intervals all facing the same direction. The standard non-woven fabric covering of the CO sensors was covered with an additional layer of 500 mesh/inch stainless steel screen for mechanical protection. The screen being just 0.05 mm thick with over 25% open area, does not materially affect the temporal response of the devices compared to the fundamental scale of the systems to be observed here. This screen was formed into a cap and fixed to the sensor with epoxy, held in place by a 3 mm tight-fitting ring of silicone tubing. The capped and silicone tubing wrapped sensors were press-fitted into 5.5 mm radius and 5 mm deep holes in the pole. Opposite each sensor, a 6 mm hole was drilled into the pole and fitted with a 6 mm o.d., 3 mm i.d. silicone tube to allow injection of gas at any of the sensor depths.

All tubing, power and signal wires were routed through the center of the pipe with a lateral exit at the top of the probe. A 5 cm long conical brass tip was fitted to the bottom to ease insertion into the snow and a removable handle for insertion and removal fitted at the top. The probe could also be pounded with a 3 kg hammer into particularly resistant snow, though great care must be taken to assure that lateral pole movement was less than that which could be closed by the taper of the poles. The poles were installed in the snow such that the uppermost sensor on the pole was just above the snow surface.

Injection of 500 ml of gas was carried out in less than 2 minutes using five injections with a 100 ml plastic syringe and two T-valves (Figure S1). To fill the syringe, the tank regulator was set to a pressure just sufficient to push out the syringe. The valves were placed in position A (Figure S1) and a silicone pinch-pipe at the first T-valve was closed to force the CO into the syringe. When the pinch-pipe was released, the syringe pressure equilibrated to the local atmosphere. The valves were then turned to position B, taking care to first turn the valve attached to the pinch-pipe to avoid forcing any excess CO into the probe. When the second valve was positioned, the syringe was emptied in a few seconds. After the valves are returned to position A, while taking care to first shut off the probe inlet, the syringe is ready for its next filling. Carbon monoxide sensors were sampled at 10Hz, recording one-minute averages of the 600 values.

Figure S1. Schematic of the tracer injection system used in the field experiment.

S2. In situ sensor calibration

A calibration of the sensors is required to translate the sensor output voltage to a corresponding trace gas concentration. Since it is essential that such a calibration is representative of the ambient conditions in which the sensor normally operates (e.g., the sensors employed here are sensitive to absolute humidity), an in situ calibration is desirable to measure the in situ behavior of the sensors (e.g., here when in contact with snow at conditions of those during the experiment). To this end, a 1.1m PVC pipe of 160 mm diameter was used as an in situ calibration chamber.

The pipe was vertically inserted in the snow pack and the four probes were pushed into the snow core with the CO sensors facing the centerline of the pipe. The top end of the pipe was sealed leak-proof with a lid while the bottom end of the pipe remained open. The calibration assembly was left in place for several hours to allow the system to equilibrate with ambient temperature and humidity. Air and 1% CO from compressed gas tank supplies were blended (Brooks 5850E series mass flow controllers) and injected at 4 l/min through the lid of the PVC pipe. CO was measured successively at blended concentrations of 5, 500, and 10'000 ppmv for approximately one hour per concentration.