Citation

Jin, G., Z. Zhang, H. Tang, Y. Xiaoquan, L. Li, and D. A. Barry. 2019. Colloid transport and distribution in the hyporheic zone. Hydrological Processes. 33(6): 932-944. Doi: 10.1002/hyp.13375.

Colloid transport and distribution in the hyporheic zone

Guangqiu Jin1, Zhongtian Zhang1, Hongwu Tang1,#, Yang Xiaoquan3,1, Ling Li2, D. A. Barry4

1 State Key Laboratory of Hydrology-Water Resources and Hydraulic Engineering, Hohai University, Nanjing, China

Emails: jingq@hhu.edu.cn, zhongtian_zhang@163.com, hwtang@hhu.edu.cn

2 School of Engineering, Westlake University, Hangzhou, China

Email: liling@westlake.edu.cn

3 Guiyang Aluminum & Magnesium Engineering & Research Institute Co., Ltd, Guiyang, 550081, China

Email: mnmn520530@126.com

4 Laboratoire de technologie écologique, Institut d’ingénierie de l’environnement, Faculté de l’environnement naturel, architectural et construit (ENAC), Ecole Polytechnique Fédérale de Lausanne (EPFL), Station 2, 1015 Lausanne, Switzerland

Email: andrew.barry@epfl.ch

# Abstract

Colloids moving from the stream into the hyporheic zone may have a negative impact on aquatic ecosystems as they are potential contaminants or carriers of contaminants. Moreover, retained colloids in the hyporheic zone could not only reduce the exchange flux between the stream and streambed but also change the conditions of the bed, affecting the habitats for aquatic organisms. Previous studies focused on the exchange flux across the sediment-water interface, but the colloid transport processes and distribution of retained colloids in the streambed has received little attention. We conducted experiments within a laboratory flume to examine these processes in a streambed driven by bedform-induced hyporheic flow. Retained colloids measured in the bed at the end of the experiments revealed colloid retention mainly in the shallow layer of hyporheic zone (0-5 cm below the interface). The results demonstrated significant effects of particle trapping and settling on the colloid transport and distribution in the streambed. Retention leads to the formation of a colloid-filled shallow layer in the bed. Particle paths based on model simulations showed that colloid settling in pore water modifies the direction of colloid transport and allows the colloid particles to move more deeply in the bed.

Keywords: Hyporheic exchange; bedform; colloid transport; colloid retention; colloid settling

# 1 Introduction

Colloids are microscopically dispersed insoluble particles of sizes between 1.0 nm to 10.0 μm, which exist widely in stream/river systems [Huettel et al., 1996]. Colloids are often associated with inorganic matter such as metal oxide, clay minerals, silicate and carbonate, or organic corrosive materials, viruses, bacteria and synthetic organic macromolecules [Pretty et al., 2006]. Sedimentation of colloids and streambed colmation impose a serious threat to the diversity and metabolism in streams [Mathers et al., 2014; Wharton et al., 2017; Partington et al.2017]. Colloids get transported into the streambed driven by hyporheic flows. Part of them may be subjected to trapping due to small pores, termed as retention [Harvey et al. 1991; Packman et al., 1996; Ma et al. 2011; Ma et al. 2013]. The retention mechanisms are mainly provided by filtration, adsorption and gravitational deposition [Harvey et al. 1991; Packman et al., 1996; Casas-Mulet et al., 2017].

Colloids retained in the streambed may contribute to reducing the bed permeability and porosity [Zheng et al. 2014; Nowinski et al., 2011; Dong et al., 2012], and the overall rate of hyporheic exchange between the overlying water and streambed. Colloid retention in the hyporheic zone of an impounded river was found to reduce the seepage rate by about 40% to 60% over a period of 4.5 years [Blaschke et al., 2003]. Retention of fine colloid particles leads to degradation and even loss of streambed habitats for associated aquatic and invertebrate species (e.g., due to changes in nutrient fluxes or in biogeochemical conditions) [Boulton, 2007]. For instance, colloid deposits near the bed surface may affect salmon spawning due to reductions in hyporheic flow speed [Malcolm et al., 2008] as well as transport of oxygen and metabolic wastes within the hyporheic zone [Malcolm et al., 2004]. In addition, pollutants adsorbed by colloids can potentially get transported more deeply into the streambed [Karathanasis, 1999, 2000]. Thus, colloid transport in streambeds and exchange with stream water are important for the ecological functioning of the hyporheic zone.

Previous studies examined the colloid exchange flux between the stream and streambed based on field investigations, laboratory experiments and numerical simulations [Packman et al., 2000a, 2000b; Packman and MacKay, 2003; Drummond et al. 2014, 2017; Jin et al. 2018]. Numerical simulations were conducted to examine hyporheic exchange and streambed filtration of suspended particles, but with no consideration of particle settling [eg. Karwan and Saiers, 2012]. Later, it was shown that  gravity was a significant driving force for colloid deposition; and experiments and numerical simulations based on a 1-D model showed important effects of particle settling on the colloid transport [Chrysikopoulos et al. 2014]. Moreover, field investigation was conducted about fine particles transport in rivers, and significant amounts of particles retained in the streambed were found [Drummond et al. 2014, 2017]. Experiments and a physicochemical model demonstrated that advection (driven by hyporheic flow associated with interactions between stream flow and bedform), particle settling, and retention (filtration) is the major process that influence the colloid exchange between the stream and streambed [Packman et al. 2000a, 2000b; Jin et al. 2018]. Particle settling enhanced colloid deposition in the hyporheic zone [Fries and Trowbridge, 2003], which in turn changed the bed properties including porosity and hydraulic conductivity [Mooneyham and Strom, 2018]. There were also simulations with considering particle aggregation effects on colloid exchange between the stream and streambed based on a pumping exchange model [Areepitak and Ren, 2011]. Control-experiments were conducted to explore the differences between beds with and without the influence of colloids deposition [Fox et al. 2018]. Colloid clogging and filtration in the hyporheic zone under different conditions have been observed in previous studies [Brunke, 1999; Blaschke et al., 2003; Hünken et al., 2007; Datry et al., 2015]. Flume experiments have also been simulated with a new numerical method multi-scale lattice Boltzmann method with respect to colloids depositing and suspending in stream and streambed systems [Chen et al. 2010]. While these studies have indicated important roles of particle settling and retention in colloid transport in the streambed, they focused on the exchange flux of colloids across the sediment-water interface and paid less attention to the actual transport colloid processes and distribution within the streambed.

Despite numerous studies in the area as reviewed above, the following questions remain: (1) What is the distribution of retained/trapped colloids in the hyporheic zone? (2) How do the retention and settling velocity control this distribution? This study aimed to address both questions by combining laboratory experiments and numerical modelling. We considered a streambed with periodic bedforms, which interact with the flow of overlying water and generate hydraulic gradients that drive pore water flow in the hyporheic zone. Colloids move in the bed with the combined velocity of pore water flow and particle settling (Fig. 1a), and are subjected to retention. Although colloid retention in the bed can potentially reduce the bed hydraulic conductivity and in turn affect the flow and the colloid transport itself, such feedback is not considered here since relatively low colloid concentrations in the stream water are applied in the experiments and numerical simulations and hence the amount of retained colloids in the bed is small. This experimental conditions follows that of Packman et al. [2000a, 2000b] and Jin et al. [2010, 2015] to allow the focus of the study on the key processes that influence colloid transport and distribution in the bed.

# 2 Laboratory experiments

## 2.1 Flume and sand

A flume experiment is a way to simulate the natural river with well-controlled conditions, by which general laws of solution or particles transport mechanism would be found. Many previous studies were based on this experimental method, e.g. [Elliott and Brooks, 1997; Packman et al., 2000(a)]. An experiment was conducted in a recirculating flume that physically models a river with a bed of uniform triangular bedforms (Fig. S1a in Supplementary Information, SI). The wall of flume is made of toughened transparent glass to permit observations through visual images. The effective length of the sand bed is about 10 m, with thickness varying between 14 and 17.5 cm from the trough to crest of the bedform, and the average overlying water depth is 11.2 cm for experiment (Table 1). A transducer was installed in the tail end of the sink for monitoring the temperature, conductivity, pH and salinity of the overlying water.

We used artificial sand supplied by Nanjing Ninglei Sand and Stone Factory (China) for experiment. The sand was washed thoroughly to remove impurities and adsorbed chemicals following a 5-step washing procedure (Section S4). The artificial sand was sieved to retain grain sizes in the range 0.25 - 0.60 mm, with a median grain size = 0.387 mm. The bulk porosity () was measured to be 0.457 with water evaporation method (Section S4), and considered as a constant over time during the experiments and simulations because the amount of retained colloids was low and did not cause much clogging of pore space. The saturated hydraulic conductivity was measured to be (K) 4.35 × 10-3 m s-1 with the constant-head method (Section S4) [Chinese National Standard, 1999; Jin et al., 2010, 2011] and also considered as a constant over time during the experiments and simulations. The river sand from the Yangtze River had the following corresponding characteristics: = 0.30 mm,  = 0.355 and K = 8.39 × 10-4 m s-1 [Jin et al., 2010, 2011, 2015].

## 2.2 Colloid properties, preparation and measurements

Kaolinite colloids (Jihong Factory, Nanjing, China) were used in the experiments. Prior to the experiments, the kaolinite was pretreated using a solution of 2M NaCl to avoid coagulation during the experiments. The kaolinite was placed in the NaCl solution and stirred for a day to convert the kaolinite to homoionic sodium kaolinite [Packman et al., 1996]. The treated kaolinite was rinsed several times using deionized water until the NaCl concentration in the rinsed solution was to 20 mM. The size distribution of the kaolinite colloids was measured after pretreatment. The results showed little changes of the colloid size over time, indicating no coagulation of colloid particles (Fig. S2) [Packman et al., 1996].

During the experiments, water samples were collected for measurements of colloid concentrations using an ultraviolet spectro photometer. We determined the light absorbance for every sample at a temperature of 20℃. The measured light absorbance was then converted to the colloid concentration based on a calibrated linear relationship between the two [Jin et al., 2014; Fig. S3].

## 2.3 Experiments

An experiment was run using the artificial sand. After the sand was packed in the flume to create the bed with the bedforms, overlying water was set to flow over the bed at a pre-set velocity to test the stability of the bedforms (Table 1). Subsequently, NaCl was added to and mixed with the overlying water to reach an initial ionic strength of 5 mM based on previous work [Packman et al., 2000]. Similarly, kaolinite was added to make an initial colloid concentration equal to 200 mg L-1 in the overlying water. Note that both NaCl and colloids were absent in the bed initially. NaCl and colloid concentrations were determined from samples collected at different times during the experiment. The experiment continued until measured NaCl and colloid concentrations in the overlying water reached (quasi-) steady values. NaCl was used as a tracer to show the transport process of a conservative solute in comparison with the behavior of colloids.

Trapped colloids within the streambed were measured at the end of the experiment. First, the pore water in the streambed was drained very slowly as controlled by a discharge gate to ensure that the original retention of colloids would not be disturbed. For the experiment, there were 32 bedforms in total: measurements were made at bedforms 2, 16, 17 and 31 from left to right. Samples of the bed sediments for experiment, 10 ml each, were collected at different depths along 8 locations/arrays (over a width of 2.70 cm each) on the stoss slope side and 3 arrays (over a width of 2.67cm each) on the lee slope side (Fig. S1b). Sampling was done layer by layer, with each layer being 1-cm thick. Layers were removed first from the bedform crest then moved progressively to the edge on both sides to avoid sand collapse. The collected sand samples were placed in test tubes with 15 ml deionized water added. The test tube with the sand sample was shaken in an ultrasonic vibration chamber for 25 min to ensure that the colloids were detached from sand particles. Fig. S4a shows the solution of detached colloids after the treatment by the ultrasonic vibration and Fig. S4b shows the effects of ultrasonic vibration on detached colloids from sands over time. The solution with detached colloids was then measured using a spectrometer for the colloid concentration, which was converted to a mass fraction of colloid in each sand sample. Finally, we also sampled sands with retained colloids on the surface of other bedforms, and made observations of the samples with a microscope, where retained colloids were found.

# 3	Numerical simulations

## 3.1	Simulation of surface water flow, and pore water flow, solute (salt) transport and colloid transport

Since different processes are involved, a one-way sequential coupling method similar to Cardenas’ [Cardenas, 2006] was used to simulate the stream water flow, and pore water flow solute transport and colloid transport in the streambed. The stream water flow was computed using the 2D CFD package, FLUENT, based on the Reynolds-Averaged Navier-Stokes equations together with the k-ω turbulence closure scheme [Jin et al., 2010]. The predicted pressures at the bedform surface were used as boundary conditions in a 2D COMSOL-based model that simulated pore water flow, solute transport and colloid transport in the streambed. The pore water flow was modeled using the combination of Darcy’s Law and the continuity equation for incompressible flow in a non-deformable medium as well as boundary conditions as described by Jin et al. [2010]. Finally, solute and colloid transport were simulated based on the pore water flow field (Figs. S5a & b).

3.1.1	Mathematical model and boundary conditions for overlying surface water

The FLUENT finite-volume based software [FLUENT, 2006] was used to simulate numerically the two-dimensional turbulent flow over the bedforms. The governing equations for the simulation were the continuity equation and Reynolds-Averaged Navier-Stokes (RANS) equations for an incompressible, homogeneous fluid. These equations, written in the Cartesian tensor notation, are:

(1)

(2)

where (kg m-3) and  (Pa s) are water density and dynamic viscosity, respectively; Ui (i = 1, 2; m s-1)is the time-averaged velocity in the xi direction (i = 1, 2 for the horizontal direction and vertical directions, respectively; m); P (Pa) is time-averaged pressure; andis the Reynolds stress tensor. Note that the gravitational potential is implicit in the pressure term of the above equation since the gravity is not expressed explicitly for a fixed boundary problem as simulated here (with the free surface approximated by a rigid, slip boundary) [Cardenas and Wilson, 2007a]. The Reynolds stresses are related to the mean rates of strain by the eddy-viscosity relation, i.e.,

(3)

where  (m2 s-1) is the kinematic eddy viscosity, k (J)is turbulent kinetic energy, and is the Kronecker delta. We adopted the k-ω turbulence closure scheme [Wilcox, 2006] since it has been demonstrated to work well for separated flows with adverse pressure gradients, including flow over dunes where pronounced eddies exist [Cardenas and Wilson, 2007b; Salehin, 2004; Yoon and Patel, 1996]. The eddy viscosity in this closure scheme is given by:

(4)

where  is the ratio of the turbulence dissipation rate  to k:

(5)

and  is a closure coefficient. The steady-state transport equations for k and  are:

(6)

(7)

where  (m2 s-1) is the kinematic viscosity of water, and the values of the various model constants are = 5/9,  = 3/40, * = 9/100 and == ½ [Wilcox, 2006].

The free surface was approximated as a frictionless and rigid lid, by specifying zero shear stresses at the fixed surface elevation (Fig. S5a). It is treated as a symmetric boundary in the FLUENT simulation package. Such an approximation of the free surface has been commonly used and demonstrated to be sound for conditions with small bedforms relative to the overlying water depth; and it enables efficient simulations with a fixed model domain [Alfrink and Van Rijn, 1983; Cardenas and Wilson, 2007b; Salehin, 2004; Yoon and Patel, 1996].

The horizontal extent of the model domain included only one bedform. We imposed spatially periodic pressure and velocity boundary conditions on the two vertical sides of the surface water domain (and the porous bed). A constant flux condition was specified in the simulation of the surface water flow. This resulted in a pressure drop, i.e., , between the two sides (Fig. S5a). Pressure was also specified at the upper right or lower right corner of the RANS domain in order to achieve a unique solution [Cardenas and Wilson, 2007c].

The sediment-water interface was treated as a no-slip wall boundary. FLUENT uses an enhanced wall treatment for the near-wall region. The treatment is based on a near-wall modelling method that combines a two-layer model (including a laminar sub-layer and a fully-turbulent outer region) with enhanced wall functions [FLUENT, 2006].

To represent accurately the geometry of the domain, we used AutoCAD to draw the bedforms and then exported the data file to FLUENT. The simulation domain was then discretized by structured grids with quadrilateral mesh elements. The grid was refined near the sediment-water interface. There were 50,000 – 160,000 elements within a single bedform in the simulations with grid sizes ranging between 0.4 mm and 9 mm. FLUENT uses a time-marching scheme to solve the steady flow problem. The iteration proceeds until a steady-state solution is reached with the maximum relative difference between computed flow variables (u, v, p, k,) of two consecutive iteration steps dropped below 10-5.

3.1.2	Mathematical model and boundary conditions for pore water

COMSOL Multiphysics, a finite element-based software [COMSOL, 2008], was used to model numerically flow and solute transport in the bed. The pore water flow is governed by the combination of Darcy’s Law and the continuity equation for incompressible flow in a non-deformable medium; hence the groundwater flow equations are:

(8)

(9)

where  is the bulk porosity of the sand bed; K (m s-1) is the hydraulic conductivity of the bed, ( (m2) is the intrinsic permeability of sand and g (m s-2) is the magnitude of gravitational acceleration); (i = 1, 2) is pore water flow velocity component in the  (i = 1, 2) direction; and h is hydraulic head, = p/g + x2 with p being the pore water pressure. Note that the above governing equations of the pore water flow are based on a constant fluid density. In other words, the effect of density variations on the flow is assumed to be small compared with that of hydraulic head gradients on the sediment-water interface generated by current-bedform interactions. Simulation parameters are in the Table 1.

3.1.3 Simulation of colloid transport using a modified transport equation

The colloid transport equation includes terms for the key processes including advection, diffusion, settling, retention and detachment [Bekhit and Hassan, 2005],

(10)

(11)

where t (s) is time, Cc (kg m-3)is the colloid concentration in pore water, Sc (kg m-3)is the total concentration of all trapped colloids (in mass per unit volume of porous media),  (i = 1, 2, m s-1) are the pore water flow velocity components in the  (i = 1, 2, m) direction, vs is the colloid settling velocity (which exists only in the vertical direction), k1 (s-1) is the colloid retention rate coefficient, kdet (s-1) is the colloid detachment rate coefficient and  (m2 s-1) are the components of the 2D dispersion coefficient tensor:

(12)

Here, and are the longitudinal and transverse dispersivities, respectively.  is the magnitude of the pore water flow velocity and  is the effective molecular diffusion coefficient.

We took the particle settling velocity into consideration so that  was combine with the  for computing the colloid advective flux. As the colloid detachment rate coefficient (kdet) is too small compared with colloid retention rate coefficient (k1) (details in SI), we neglected the detachment term to obtain a simplified governing equation for colloid transport:

(13)

(14)

Note that salt solute transport can be simulated using Eq. (13) with the particle settling and retention terms removed.

## 3.2 Settling velocity vs and retention rate coefficient k1

To determine the model parameters values, a column experiment was also conducted (Fig. S6a). The kaolinite solution flows through the column packed with washed sand (height 20 cm, diameter 5 cm) from bottom to the top under a constant head condition. The colloid concentration of the effluent was measured continuously during the experiments. Three experiments were conducted under different average pore water velocity (u), 8.92×10-4 m s-1, 5.02×10-4 m s-1 and 2.30×10-4 m s-1, respectively. The initial kaolinite concentration was set to 200 mg L-1 .

The larger the flow rate is in the column, the greater the breakthrough colloid concentration () is. According to the classic filtration theory, an empirical filtration coefficient of the colloid trapping is given below [e.g. Tobiason and O’Meli, 1988; Ren et al., 2002]:

(15)

(16)

where C (kg m-3) and C0 (kg m-3) are the concentrations at the outlet concentration and input concentration, respectively, in the column experiment; and x (m) is the length of the column; λ (m-1) is the filtration coefficient. The retention rate coefficient k1 is then given by [Ren et al., 2002]:

(17)

where, u (m s-1) is the pore water velocity. Based on Eq. 17 and results of three column experiments, we determined the relationship between retention rate coefficient and pore water flow velocity (Fig. S6b). Using this relationship, we estimated the value of retention rate coefficient (k1) around 2.4×10-3 s-1 for the flume experiments with pore water flow velocity varying between 1×10-5 and 2×10-5 m s-1.

The determination of settling velocity is also important for modelling colloid transport in the bed. For this purpose, the model was calibrated against the experimental data with the settling velocity as a fitting parameter. Details of model calibration are given in the Supporting Information (SI). With vs=2.0×10-5 m s-1, the model predictions (shown in Figs. 2b & 5a) was found to match the data the best, with the largest coefficient of correlation. The estimated model parameter values from model calibrations were used in the subsequent simulations of the flume experiments (Table 1).

## 3.3 Boundary conditions for colloid transport

The boundary conditions for the colloid transport were set as follows (FigS. 6b): (1) on the lateral boundaries (= 0, and = L) of the domain, periodic conditions with Cc(0,,t) = Cc(L,,t) and  were imposed. Note that only one bedform was simulated with processes and conditions assumed to be invariant between different bedforms; (2) on the bottom of the domain, a no-flux condition applies, giving a zero colloid concentration gradient  since no local vertical flow exists and hence the advective flux is zero at this boundary; (3) along the sediment-water interface, the boundary condition was set as follows:

(19)

where n is the unit vector normal to the interface (pointing inward), u is the colloid particle flow velocity vector, =ui - vs ; and  (kg m-3) is the colloid concentration in the overlying water at time t. It should be noted that the overlying water in our experiments was found to be relatively well mixed and hence a spatially uniform concentration was assumed along the long flume. However,  varied with time as a result of the mass exchange between the overlying water and the bed, and was calculated as follows:

(20)

where  (kg m-3)is the initial colloid concentration of the overlying water,  (m3) is the total volume of water in the flume system excluding pore water in the streambed,  and  are the simulated concentrations of colloids in the pore water and trapped colloids in the sand at location and time t, respectively, B (m)is the width of the flume, and A (m2) is the area (on the  plane shown in Fig. S6b) of the bed in the flume. The integral in equation (20) represents the amount of colloids retained in the bed.

# 4 Results and Discussion

In this paper, we assumed that the retained colloids would not have significant feedback effect. The feedback refers to the fact that the former retained colloids would change the porosity and hydraulic conductivity, which may in turn affect the transport of later coming colloids. The rational is presented in Section S2.

## 4.1 Variations of colloid concentration in the overlying water

In the overlying water, the colloid concentration in the experiment decreased quickly during the first day (24 h), as shown in Fig. 2a. The concentration decline was less rapid on the second day. The concentration decreased to nearly zero towards the end of the experiment (72 h), indicating an almost total removal of colloids from the stream and pore water as a result of colloid retention by sand particles. In contrast, the NaCl concentration decreased relatively slowly and approached a constant, non-zero value at the end of the experiment. NaCl, as a solute, remained in the stream water and pore water. The decline of the NaCl concentration in the overlying water reflects the mass transfer of NaCl to the pore water, without physical retention as in the case of colloids.

The numerical model simulated well the measured concentration changes in the overlying water for colloid (Fig. 2a) as the coefficients of correlation (R) is 0.9854 (Table S1), but the model overestimated before 10 h and underestimated afterwards. The model parameter values are according to measured sand properties, stream water flow velocities and sizes of the bedforms, and the estimated particle settling velocity and retention rate coefficient from model calibration (Table1).

With different settling velocities, the simulated colloid concentration in the overlying water varied greatly, because the settling velocity directly influences the colloid influx to the bed. As the velocity of particle movement is given by u =ui - vs , a higher settling velocity leads to a larger colloid influx (downward) and hence more rapid removal of colloids from the overlying water and more rapid decline of the concentration as shown in Fig. 2b. As the true settling velocity (vs) is unknown, vs was determined based on the maximum coefficient of correlation (R) (Section 4.3 and Table S1).

Using the model, we conducted further simulations to explore the effects of the retention rate coefficient and settling velocity on the colloid transport. Three different retention rate coefficients were examined. The simulated colloid concentrations in the overlying water exhibited almost no difference among the simulations with the three different retention rate coefficients (Fig. 2c). This is because changes of the retention rate coefficient did not affect the influx of colloids to the bed, and colloids were retained subsequently. These results also suggest that all the colloids that moved into the bed were retained fully even at the lowest retention rate tested. Hence, the decline of the colloid concentration in the overlying water largely depended on the mass transfer rate of colloids from the stream to the bed and was affected little by the retention rate coefficient (Fig. 2b & c).

## 4.2 Variations of colloid concentration in the pore water

As the colloid concentrations in pore water were too low to measure, we examined the simulated results, focusing on the total mass in one bedform and concentrations (Fig. 3c) at two observation points shown in Fig. 3b (P1 and P2). The results show a phase of total mass rise sharply to a peak value at the beginning (t = 2000 s) followed by a very slow decline phase with the mass reaching zero at the end. Similar trends could be seen in the concentration variations at both observation points. Note the logarithmic scale used in Fig. 3c. The rising phase reflects the arrival of water initially of high colloid concentration. As more and more colloids were retained/trapped in the bed, colloid concentrations in water both above and inside the bed declined eventually to zero as shown in Fig. 3c. We examined the variation of colloid concentration in pore water over depth at t = 2000 s, which exhibited a decreasing trend (Fig. S7). This trend is similar to that of retained colloid concentration over depth (Fig. 4a & b), but with much smaller concentration values.

Along the particle travel path that went through the two observation points (indicated by the red line Fig. 3d), we analyzed the peak colloid concentration in pore water (Fig. 3d). As the distance (Ls) from the starting point of the path increased (i.e., moving downward along the path), the peak concentration decreased rapidly and occurred at increasingly longer elapsed time. To examine the colloid distribution in pore water over one bed form at different elevation (y), we integrated the pore water colloid concentration horizontally over the extent of the bedform at y, i.e.,

(21)

where Cc is the colloid concentration in pore water, B is the width of the flume and L defines the extent of the bedform at different y (varying with y for y>0 but becoming constant for y<0). Mc (kg m-1) represents the colloid mass distribution per unit height in pore water over one bedform. The results calculated for t=2×102 s, t=2×103 s, t=2×104 s and t=2×105 s showed similar distribution patterns for the first three elapsed time, with the peak value around y=0 m where the extent of the bedform reaches the maximum (Fig. 3e). Due to retention, less and less colloid particles reached the depth below y=0 m. The peak value of Mc initially increased with time but subsequently the trend was reversed. By t=2×105 s, no colloids were found in pore water, indicating full retention of colloid particles by the medium. The full removal of colloid particles from water was also evident in the overlying water where the colloid concentration became zero (Fig. 2a).

## 4.3 Colloid retention in bedforms

At the end of the experiment, sand samples were examined under a microscope, and the images of sands from a streambed in our flume experiment on colloid retention show clearly evidence of retained colloids on the sand surface (Fig. 1b & c). To analyze the colloid retention in the streambed, we calculated the total mass of colloids trapped in the bedforms based on concentrations of retained colloid particles measured at the end of the experiment. The total mass of colloids trapped in the entire bed was found to be close to the amount of colloids initially added to the stream water. Almost all colloids were retained – 98.5% of colloid particles initially put to the stream water retained in the bed according to measurements and 99.9% retained as predicted by the model simulation (more details in Section S2).

The measurements also show that colloids were mainly retained in the shallow layer of the bed close to the streambed surface (Fig. 4a & Fig. S8). The dominating retention area was within a depth below water-sediment interface of 5 cm on the stoss slope side and only 2 cm on the lee slope side. This result indicates that retention played a major role in determining the colloid distribution in the streambed. The amount of trapped colloids on the stoss slope side was more than that on the lee side (Fig. 4a). The model simulated well the observed colloid distribution pattern in the bed (Fig. 4b). The colloid distribution in the streambed was shown to be significantly affected by the settling velocity. In particular, with no settling or low settling velocities, the model failed to predict the presence of trapped colloids below the surface of the lee slope (Fig. 4c). Further simulation with settling velocity set to zero showed that if there was not settling effect of colloid particles, the lee side of the bedform would contain no retained colloids from x = 0.24 m to x = 0.28 m (Fig. 4c). Colloid particle settling in pore water affects significantly the colloid transport in the bedforms, modifying the paths of colloid particle movement and distributions of trapped colloids. Previous studies [Packman et al., 2003; Rehg et al., 2005; Ren et al., 2005] showed that colloids could get trapped on the out-flow side (lee side) of bedforms. Clogging of the lee side of bedform by colloids was clearly evident in the experiments of Rehg et al. [2005]. Particle settling enabled colloids to travel to a wider area on the lee side of bedforms (Fig. 4a & b).

As shown in Fig. 3, the settling velocity influences significantly the transport paths of colloids, which deviated from the pathlines given by the flow field. A relatively large settling velocity allowed colloids to reach and get retained in deep areas in the bed. More generally, increase in colloid particle settling velocity led to deeper colloid penetration into the streambed, but reduced colloid retention in the upper layer (Fig. S9).

The colloid distribution in the bed was found to vary greatly with the retention rate coefficient. With a lower retention rate coefficient, colloids moved more deeply into the bed, resulting in lower concentrations of retained colloids over a larger (deeper) area (Fig. S10). Different clogging patterns associated with different distributions of trapped colloids would affect the biogeochemical processes in the bed and in turn the habitat conditions [Drummond et al. 2018].

To examine the distribution of retained colloids over one bed form at different heights (y), we integrated the retained colloid concentration horizontally over the extent of the bedform at y, i.e.,

(22)

where Sc is the retained colloid concentration. Again L, defining the extent of the bedform at different y, varies with y for y>0 but becomes constant for y<0. Ms (kg m-1) represents the retained colloid mass distribution per unit height over one bedform. The results obtained at the end of the experiment and simulations with different parameters values showed similar trends (Fig. 5). The mass per unit height of retained colloids over one bedform increased with the depth and reached the maximum around y=0 m, where the extent of the bedform (L) was also at the maximum due to the triangular bedform. Below y=0 m, the mass of retained colloids decreased dramatically with the depth. An increase in the settling velocity led to deeper colloid penetration into the streambed with the peak of Ms shifted downward slightly but reduced in value, indicating less retention in the upper layer (Fig. 5a). With a lower colloid retention rate, colloids moved more deeply into the bed, resulting in similarly less colloid retention in the upper layer (Fig. 5b). The coefficients of correlation (R) of each case are shown in Table S1, and combined with the R value of concentration in overlying water, the average R was calculated, and the final settling velocity (vs) was determined to be 2×10-5 m s-1.

Comparison between the results shown in Fig. 5 and Fig. 3e showed that the magnitude of the total colloid mass in pore water at the peak time (t = 2000 s) of pore water colloid concentration was almost 2 order less than that of the retained mass, despite similar trends of the mass distribution over the height for colloids in both forms.

## 4.4 Colloid mass variations in all phases

The total mass of colloid remained unchanged during the experiments but was distributed in different phases in different amounts at different times (Fig. 6). The amount of colloids in pore water was small and accounted for a very low proportion, which initially increased but then decreased to zero. The mass of retained colloids in the bed increased rapidly at the beginning but this increasing trend slowed down and eventually ceased. Overall the changes in the mass of retained colloids mirrored the reduction of the colloid mass in the overlying water as expected (Fig. 6 & Fig. 2a).

# 5 Concluding remarks

This study, through experiments and numerical simulations, demonstrated the importance of colloid retention and settling in controlling colloid transport and distribution in the streambed. Key findings are:

(1) The model predicted reasonably well the experimental results, including the effects of major parameters. The variations of colloid concentration in the overlying water were found to be sensitive to colloid settling velocity – the larger the settling velocity is, the more rapidly the concentration drops. The distribution of trapped colloids in the bedform was sensitive to both the retention rate coefficient and colloid settling velocity. A smaller retention rate or a larger settling velocity leads to more colloids trapped in the deep area of the bed.

(2) The colloid settling velocity can have significant effects on the penetration of colloids into the bedform, strongly enhancing the colloid influx to bed. The settling process enables transport of colloids into the lee-side of the bedform and more deeply in the bed, extending the zone of colloid trapping with significant modification of colloid distribution in the streambed.

(3) Colloid retention resulted in colloid deposition mainly in the shallow layer near the bed surface. This effect would lead to clogging of pore space and accelerate colloid trapping. Within the present study, the experiments and numerical simulations were conducted under relatively low initial colloid concentrations in the overlying water; and thus the amount (0.0275% of total porosity) of trapped colloids in the bed was small and did not cause much reduction in the porosity and hydraulic conductivity of the porous bed. Thus the feedback of colloid retention on the hyporheic flow and transport processes was not considered.

This study has provided new insights into colloid transport and distribution in streambeds with bedforms. The findings have implications for studies of the river system, in particular, in the hyporheic zone. In reality, clogging of the bed would likely occur over longer time scales, which would potentially lower the permeability of the medium and reduce pore water flow significantly; but this effect is not considered in this paper.

Therefore, further investigations should be carried out to examine questions concerning, for example, how colloids may affect the fluid density and so induce density-dependent pore water flow, and how colloid retention may lead to clogging, which in turn changes the porosity and hydraulic conductivity of the medium, and consequently the flow and colloid transport in the streambed.

# Acknowledgments

This research has been supported by the Natural Science Foundation of China (51421006, 51239003, 51679065, 51109059), the Basic Research Programs (Natural Science Foundation) of Jiangsu Province (BK20171436) and the 111 Project (B17015), Ministry of Education and State Administration of Foreign Experts Affairs, P. R. China..

# References

Alfrink, B. J., & Van Rijn, L. C. (1983). Two-equation turbulence model for flow in trenches. Journal of Hydraulic Engineering, 109(7), 941-958.

Areepitak, T., & Ren, J. (2011). Model simulations of particle aggregation effect on colloid exchange between streams and streambeds. Environmental science & technology, 45(13).

Bekhit, H. M., & Hassan, A. E. (2005). Two-dimensional modeling of contaminant transport in porous media in the presence of colloids. Advances in water resources, 28(12), 1320-1335.

Blaschke, A. P., Steiner, K., Schmalfuss, R., Gutknecht, D., & Sengschmitt, D. (2003). Clogging processes in hyporheic interstices of an impounded river, the Danube at Vienna, Austria. International Review of Hydrobiology, 88(34), 397–413.

Brunke, M. (1999). Colmation and depth filtration within streambeds: Retention of particles in hypoheic interstices. International Review of Hydrobiology, 84(2), 99–117.

Boulton, A. J. (2007). Hyporheic rehabilitation in rivers: restoring vertical connectivity. Freshwater Biology, 52(4), 632-650.

Cardenas, M. B. R. (2006). Dynamics of fluids, heat and solutes along sediment-water interfaces: a multiphysics modeling study (Doctoral dissertation, New Mexico Institute of Mining and Technology).

Cardenas, M. B., & Wilson, J. L. (2007a). Effects of current–bed form induced fluid flow on the thermal regime of sediments. Water Resources Research, 43(8).

Cardenas, M. B., and J. L. Wilson (2007b), Dunes, turbulent eddies, and interfacial exchange with permeable sediments, Water Resources Research, 43(8).

Cardenas, M. B., & Wilson, J. L. (2007c). Hydrodynamics of coupled flow above and below a sediment–water interface with triangular bedforms. Advances in water resources, 30(3), 301-313.

Casas-Mulet, R., Alfredsen, K. T., McCluskey, A. H., & Stewardson, M. J. (2017). Key hydraulic drivers and patterns of fine sediment accumulation in gravel streambeds: A conceptual framework illustrated with a case study from the Kiewa River, Australia. Geomorphology, 299, 152-164.

Chen, C., Packman, A. I., Zhang, D., & Gaillard, J. F. (2010). A multi‐scale investigation of interfacial transport, pore fluid flow, and fine particle deposition in a sediment bed. Water Resources Research, 46(11).

Chinese National Standard (1999). Standard for soil test method. GB/T50123-1999, Beijing (in Chinese).

Chrysikopoulos, C. V., & Syngouna, V. I. (2014). Effect of gravity on colloid transport through water-saturated columns packed with glass beads: modeling and experiments. Environmental science & technology, 48(12), 6805-6813.

COMSOL (2008). COMSOL 3.5 COMSOL Multiphysics - Modeling Guide, COMSOL Inc, Burlington, MA, USA.

Datry, T., Lamouroux, N., Thivin, G., Descloux, S., & Baudoin, J. M. (2015). Estimation of sediment hydraulic conductivity in river reaches and its potential use to evaluate streambed clogging. River Research and Applications, 31, 880–891.

Dong, W., Chen, X., Wang, Z., Ou, G., & Liu, C. (2012). Comparison of vertical hydraulic conductivity in a streambed-point bar system of a gaining stream. Journal of Hydrology, 450–451, 9–16. https://doi.org/10.1016/j.jhydrol.2012.05.037

Drummond, J. D., Davies-Colley, R. J., Stott, R., Sukias, J. P., Nagels, J. W., Sharp, A., & Packman, A. I. (2014). Retention and remobilization dynamics of fine particles and microorganisms in pastoral streams. Water Research, 66C, 459–472.

Drummond, J. D., Larsen, L. G., Gonzalez-Pinzon, R., Packman, A. I., & Harvey, J. W. (2017). Fine particle retention within streamstorage areas at base flow and in response to a storm event.  Water Resources Research, 53, 5690–5705.

Drummond, J. D., Larsen, L. G., González‐Pinzón, R., Packman, A. I., & Harvey, J. W. (2018). Less Fine Particle Retention in a Restored Versus Unrestored Urban Stream: Balance Between Hyporheic Exchange, Resuspension, and Immobilization. Journal of Geophysical Research: Biogeosciences, 123(4), 1425-1439.

Elliott, A. H., & Brooks, N. H. (1997). Transfer of nonsorbing solutes to a streambed with bed forms: Laboratory experiments. Water Resources Research, 33(1), 137-151.

FLUENT (2006). FLUENT 6.3 Documentation, Fluent Inc., Lebanon, NH.

Fox, A., Packman, A. I., Boano, F., Phillips, C. B., & Arnon, S. (2018). Interactions between suspended kaolinite deposition and hyporheic exchange flux under losing and gaining flow conditions. Geophysical Research Letters, 45(9), 4077-4085.

Fries, J. S., & Trowbridge, J. H. (2003). Flume observations of enhanced fine-particle deposition to permeable sediment beds. Limnology and Oceanography, 48(2), 802–812.

Harvey, R. W., & Garabedian, S. P. (1991). Use of colloid filtration theory in modeling movement of bacteria through a contaminated sandy aquifer. Environmental science & technology, 25(1), 178-185.

Huettel M, W Ziebis, S Forster. (1996) Flow-induced uptake of particulate matter in permeable sediments. Limnol Oceanogr, 41(2):309-22.

Hünken, A., & Mutz, M. (2007). Field studies on factors affecting very fine and ultra fine particulate organic matter deposition in low-gradient sand-bed streams. Hydrological Processes, 533(21), 525–533.

Jin, G., H. Tang, B. Gibbes, L. Li, and D. Barry (2010). Transport of nonsorbing solutes in a streambed with periodic bedforms, Advances in water resources, 33(11), 1402–1416.

Jin, G., Tang, H., Li, L., & Barry, D. A. (2011). Hyporheic flow under periodic bed forms influenced by low‐density gradients. Geophysical Research Letters, 38(22).

Jin G, Xu H, Tang H, Yang X, Zhang P, Li L. (2014). Flume methods for colloids transport in hyporheic zone. Chinese Patent, ZL201410153812.X.

Jin, G. Q., H. W. Tang, L. Li, and D. A. Barry (2015). Prolonged river water pollution due to variable-density flow and solute transport in the riverbed, Water Resources Research, 51(4), 1898-1915.

Jin, G., Jiang, Q., Tang, H., Li, L., & Barry, D. A. (2018). Density effects on nanoparticle transport in the hyporheic zone. Advances in Water Resources, 121, 406-418.

Karathanasis, A. D. (1999). Subsurface migration of copper and zinc mediated by soil colloids. Soil Science Society of America Journal, 63(4), 830-838.

Karathanasis, A. D. (2000). Colloid‐mediated transport of Pb through soil porous media. International journal of environmental studies, 57(5), 579-596.

Karwan, D. L., & Saiers, J. E. (2012). Hyporheic exchange and streambed filtration of suspended particles. Water Resources Research, 48(1).

Ma, H., Hradisky, M., & Johnson, W. P. (2013). Extending applicability of correlation equations to predict colloidal retention in porous media at low fluid velocity. Environmental science & technology, 47(5), 2272-2278.

Ma, H., Pazmino, E. F., & Johnson, W. P. (2011). Gravitational settling effects on unit cell predictions of colloidal retention in porous media in the absence of energy barriers. Environmental science & technology, 45(19), 8306-8312.

Malcolm, I. A., Soulsby, C., Youngson, A. F., Hannah, D. M., McLaren, I. S., & Thorne, A. (2004). Hydrological influences on hyporheic water quality: implications for salmon egg survival. Hydrological Processes, 18(9), 1543-1560.

Malcolm, I., Greig, S. M., Youngson, A. F., & Soulsby, C. (2008). Hyporheic influences on salmon embryo survival and performance. In Salmonid spawning habitat in rivers: physical controls, biological responses, and approaches to remediation. American Fisheries Society.

Mathers, K. L., Millett, J., Robertson, A. L., Stubbington, R., & Wood, P. J. (2014). Faunal response to benthic and hyporheic sedimentation varies with direction of vertical hydrological exchange. Freshwater Biology, 59(11), 2278-2289.

Mooneyham, C., & Strom, K. (2018). Deposition of Suspended Clay to Open and Sand‐Filled Framework Gravel Beds in a Laboratory Flume. Water Resources Research, 54(1), 323-344.

Nowinski, J. D., Cardenas, M. B., & Lightbody, A. F. (2011). Evolution of hydraulic conductivity in the floodplain of a meandering river due to hyporheic transport of fine materials. Geophysical Research Letters, 38(1), 2–6. https://doi.org/10.1029/2010GL045819

Packman, A. I., N. H. Brooks, and J. J. Morgan (1996). Experimental techniques for laboratory investigation of clay colloid transport and filtration in a stream with a sand bed, paper presented at International Symposium on the Interactions Between Sediments and Water, Baveno, Italy, Sep 22-26.

Packman, A. I., N. H. Brooks, and J. J. Morgan (2000a). Kaolinite exchange between a stream and streambed: Laboratory experiments and validation of a colloid transport model, Water Resources Research, 36(8), 2363-2372.

Packman, A. I., Brooks, N. H., & Morgan, J. J. (2000b). A physicochemical model for colloid exchange between a stream and a sand streambed with bed forms. Water Resources Research, 36(8), 2351-2361.

Packman, A. I., and J. S. MacKay (2003). Interplay of stream-subsurface exchange, clay particle deposition, and streambed evolution, Water Resources Research, 39(4).

Partington, D., Therrien, R., Simmons, C. T., & Brunner, P. (2017). Blueprint for a coupled model of sedimentology, hydrology, and hydrogeology in streambeds. Reviews of Geophysics, 55(2), 287-309.

Pretty, J. L., Hildrew, A. G., & Trimmer, M. (2006). Nutrient dynamics in relation to surface–subsurface hydrological exchange in a groundwater fed chalk stream. Journal of Hydrology, 330(1-2), 84-100.

Rehg K J, Packman A I, Ren J H.(2005), Effects of suspended sediment characteristics and bed sediment transport on streambed clogging . Hydrological Processes, 19 (2): 413-427.

Ren, J., & Packman, A. I. (2002). Effects of background water composition on stream–subsurface exchange of submicron colloids. Journal of Environmental Engineering, 128(7), 624-634.

Ren J, Packman A I. (2005). Coupled Stream-Subsurface Exchange of Colloidal Hematite and Dissolved Zinc, Copper, and Phosphate, Environmental science & technology, 39, 6387-6394.

Salehin, M. (2004). Hydrodynamics of hyporheic exchange for complex natural streambed topography, channel geometry and sediment structure, Ph. D. thesis, Northwestern University, USA.

Tobiason, J. E., & O'melia, C. R. (1988). Physicochemical aspects of particle removal in depth filtration. Journal‐American Water Works Association, 80(12), 54-64.

Wharton, G., Mohajeri, S. H., & Righetti, M. (2017). The pernicious problem of streambed colmation : A multi-disciplinary reflection on the mechanisms, causes, impacts, and management challenges. WIREs Water, e1231, 1–17.

Wilcox, D. (2006). Turbulence modeling for CFD, DCW industries La Canada, California.

Yoon, J. Y., & Patel, V. C. (1996). Numerical model of turbulent flow over sand dune. Journal of Hydraulic Engineering, 122(1), 10-18.

Zheng, X. L., Shan, B. B., Chen, L., Sun, Y. W., & Zhang, S. H. (2014). Attachment–detachment dynamics of suspended particle in porous media: Experiment and modeling. Journal of Hydrology, 511, 199-204.

FIGURES AND TABLE

White patches show retained colloids on the sand surface

(c)

Free colloids

Retained colloid

Retained colloid

Velocity of pore water

Velocity of colloid

Settling velocity

Sand

(a)

(b)

Moving colloid

Fig. 1 (a) Colloid transport processes in the streambed. (b) Microscope images of colloids retained by a sand grain. (c) Zoomed-in image of retained colloids by a sand grain.

(a)

(b)

(c)

Fig. 2 (a) Comparison of colloid and NaCl concentrations normalized by the initial concentrations (i.e., C/C0) in the overlying water. (b) Comparison of experimental and simulated colloid concentrations in the overlying water for different settling velocities.vs=2×10-5 m s-1is set for the simulation of the flume experiment. (c) Comparison of experimental and simulated colloid concentrations in the overlying water for different colloid retention rate coefficient.k1=2.4×10-3 s-1 is set for the simulation of the flume experiment. The three modelling results overlap.

(a)

(b)

(c)

(d)

(e)

Fig. 3 Streamlines in one bedform for pore water (a) and colloids (b) based on simulation results. The blue arrows represent the flow and path directions. The red line in (b) is the path selected for the analysis presented in Fig. 3d. Points P1 and P2 are for observations of colloid concentration variations in Fig. 3c. (c) Total mass of colloids in pore water over one bedform and pore water colloid concentrations at two observation points (shown in Fig. S5) based on simulation results. (d) Peak concentration of colloids in pore water and peak concentration time along the particle travel path (shown by the red line in Fig. S5b). (e) Comparison of masses per unit height of colloids in pore water per at different elevation (y) for different times. y = 0 indicates the trough elevation of streambed, positive section shows the height of bedform and negative section shows the depth in the streambed.

(a)

(c)

(b)

Fig. 4 Retained colloid concentration contours across the bedform for Case 1, in the unit of kg m-3, based on measurements (a), simulation of the flume experiment (b) and simulation without consideration of particle settling velocity, i.e., vs = 0 (c).

(b)

(a)

Fig. 5 (a) Comparison of retained colloid mass per unit height with different settling velocities (vs=2×10-5 m s-1 is set for the simulation of the flume experiment). (b) Comparison of retained colloid mass per unit height with different retention rates (k1=2.4×10-3 s-1 is set for the simulation of the flume experiment).

Fig. 6 Proportions of colloid masses in the three phrases (retention phrase, overlying water phrase and pore water phrase) varying with time.

# Table 1 Parameters values applied in the experiment and numerical simulations.

| Parameters | Experiment | Simulation |
| --- | --- | --- |
| Average velocity for overlying water (U) | 10.45 cm s-1 | 10.45 cm s-1 |
| Average water depth for overlying water (H) | 11.2 cm | 11.2 cm |
| Initial colloid concentration in overlying water () | 0.2 kg m-3 | 0.2 kg m-3 |
| Bedform length (L) | 0.3 m | 0.3 m |
| Stoss length () | 0.22 m | 0.22 m |
| Lee length (Ll) | 0.08 m | 0.08 m |
| Bedform height () | 3.5 cm | 3.5 cm |
| Streambed depth () | 14 cm | 14 cm |
| Hydraulic conductive (K) | 4.35×10-3 m s-1 | 4.35×10-3 m s-1 |
| Retention rate coefficient (k1) | 2.4×10-3 s-1 | 1.4×10-3 s-1 ; 2.4×10-3 s-1 ; 3.4×10-3 s-1 |
| Bulk porosity (θ) | 0.457 | 0.457 |
| Water density (ρ) | 1000 kg m-3 | 1000 kg m-3 |
| Water dynamic viscosity (μ) | 0.001 | 0.001 |
| Longitudinal dispersivity () | 0.001m | 0.001m |
| Transverse dispersivity () | 0.0001 m | 0.0001 m |
| Effective diffusion coefficient () | 10-9 m2 s-1 | 10-9 m2 s-1 |
| Settling velocity (vs) | Unknown | 0 ; 1×10-5 m s-1 ; 2×10-5 m s-1 (close to experiment) ; 3×10-5 m s-1 ; 4×10-5 m s-1 |

# Author to whom all correspondence should be addressed. Tel: +86 (25) 8378-6662. Fax: +86 (25) 8373-5375