Alkoxy hydrosilanes as surrogates of gaseous silanes for hydrosilylation of alkenes

Ivan Buslov, Sébastien Carlos Keller and Xile Hu *

Laboratory of Inorganic Synthesis and Catalysis, Institute of Chemical Sciences and Engineering, Ecole Polytechnique Fédérale de Lausanne (EPFL), ISIC-LSCI, BCH 3305, Lausanne 1015 (Switzerland)

Supporting Information Placeholder

[]

ABSTRACT: Me2SiH2, MeSiH3 and SiH4 are gaseous and flammable silanes that are inconvenient to use in chemical reactions. Catalytic amounts of a nickel pincer complex and NaOtBu are reported to allow the synthesis of alkyl hydrosilanes from alkenes and alkoxy hydrosilanes, leading to the replacement of Me2SiH2, MeSiH3 and SiH4 by Me2(MeO)SiH, Me(EtO)2SiH, and (MeO)3SiH in hydrosilylation reactions of alkenes. The scope and mechanism of the reactions are also described.

Silanes such as Me2SiH2, MeSiH3 and SiH4 (boiling points of -20˚C, -57˚C and -112˚C, respectively) are flammable gases at room temperature and are inconvenient to handle on a laboratory scale.1a Moreover, SiH4 is pyrophoric and causes severe safety concerns. 1b As a result, these silanes are rarely applied in the hydrosilylation of alkenes,2  which is one of the most important methods to produce silicon coupling reagents, silicon polymers, and organosilicon compounds for fine chemical synthesis.3 Oestreich and co-workers recently developed an elegant approach using 3-silylated cyclohexa-1,4-dienes, activated by B(C6F5)3, as precursors for gaseous hydrosilanes (eq. 1-2, Scheme 1).4 However, the scope and tolerance of this method remain modest due to the reactive B(C6F5)3 catalyst. Herein we report that readily available alkoxy hydrosilanes can be used as surrogates of Me2SiH2, MeSiH3 and SiH4 in Ni-catalyzed hydrosilylation of alkenes (eq. 3, Scheme 1), yielding alkyl hydrosilanes otherwise difficult to access. Broad scope and good functional group compatibility have been achieved.

Scheme 1. The development of surrogates of gaseous silanes

While studying Ni-catalyzed hydrosilylation of alkenes,5 we tested different nickel complexes for hydrosilylation using tertiary silanes. Unexpectedly, in the reaction of 1-octene with Ph2(OMe)SiH using the nickel amido(bisoxazoline) complex, [iPr2-(S,S)-Bopa]NiCl (1a) as pre-catalyst and NaOtBu as base, Ph2(Octyl)SiH was isolated in a 81% yield (relative to 1-octene, eq. 4). The conventional hydrosilylation product, Ph2(Octyl)(OMe)Si, was not formed. Recognizing that this unusual transformation might allow the replacement of certain silanes by alkoxy hydrosilanes in hydrosilylation reactions, we decided to further explore it.

The reaction of 1-decene with Me2(MeO)SiH was chosen as the test reaction for the optimization of conditions (Table 1).6 In tetrahydrofuran (THF) and at room temperature, (Decyl)(Me)2SiH was formed in a 91% yield using 2.5 mol% of 1a as catalyst and 5 mol% of NaOtBu as base (entry 1, Table 1). Me2(MeO)2Si was the byproduct. If NaOtBu was replaced by CsF, the yield was less than 1% (entry 2, Table 1). The yields were lower when THF was replaced by toluene or diethyl ether (entries 3-4, Table 1). When 1a was replaced by its phenyl analogue, [Ph2-(R,R)-Bopa]NiCl (1b), the yield dropped to 59% (entry 5, Table 1). Two other Ni pincer complexes, 1c and 1d, were also tested as catalysts. However, the yields were below 50% (entries 6-7, Table 1). A soluble Ni(II) complex, NiCl2(PPh3)2 was also tested as catalyst, but again the yield was modest (entry 8, Table 1). These results confirmed the superior catalytic activity of 1a in this reaction. When the loading of 1a was reduced to 1 mol%, the yield was nearly the same (87%; entry 9, Table 1). When 1 equivalent of NaOtBu relative to 1a was added, no (Decyl)(Me)2SiH was formed (entry 10, Table 1). When NaOtBu was replaced by KOtBu, the yield was similar (Entry 11, Table 1).

Table 1. Optimization of reaction conditions a

| entry | complex, mol % | base, mol % | solvent | yield, %b |
| --- | --- | --- | --- | --- |
| 1 | 1a, 2.5 | NaOtBu, 5 | THF | 91 |
| 2 | 1a, 2.5 | CsF, 5 | THF | <1 |
| 3 | 1a, 2.5 | NaOtBu, 5 | toluene | 45 |
| 4 | 1a, 2.5 | NaOtBu, 5 | Et2O | 31 |
| 5 | 1b, 2.5 | NaOtBu, 5 | THF | 59 |
| 6 | 1c, 2.5 | NaOtBu, 5 | THF | 40 |
| 7 | 1d, 2.5 | NaOtBu, 5 | THF | 48 |
| 8 | NiCl2(PPh3)2, 2.5 | NaOtBu, 7.5 | THF | 41 |
| 9 | 1a, 1 | NaOtBu, 2 | THF | 87 |
| 10 | 1a, 2.5 | NaOtBu, 2.5 | THF | 0 |
| 11 | 1a, 2.5 | KOtBu, 5 | THF | 90 |

a Conditions : 1-decene (0.5 mmol), dimethylmethoxysilane (1.2 mmol.), under N2 in THF (4 mL), 12 hours, rt. b GC-MS yields are reported and n-dodecane was used as an internal standard. The yield is relative to 3a.

The optimized conditions were applied for the reactions of Me2(MeO)SiH with various terminal alkenes (Scheme 2). 1-Decene and 1-octadecene were hydrosilylated with high yields (4a, 4b). 2-Methyl-2-heptene, which contained a methyl group at the β-position of the alkene, was also converted to the corresponding hydrosilane in a good yield (4c). The reaction was selective to terminal C=C bond while leaving an internal C=C bond intact (4e). Functional groups such as ether (4f), epoxide (4g), tert-butyldimethylsilyl- (4h) and tetrahydropyran-protected alcohol (4i), alkyl halide (4j), amine (4k) and acetal (4l) were tolerated in the catalysis. Unfortunately alkenes bearing carbonyl, ester, amide and imide groups were not suitable reaction partners, likely due to the high reducing power of silanes in the presence of a base.7, 8

Scheme 2. Ni-catalyzed synthesis of alkylhydrosilanes using 2a and alkenes a

a Conditions : alkene (0.5 mmol), dimethylmethoxysilane (1.2 mmol.), 1a (2.5 mol %), NaOtBu (5 mol %), THF (3 mL), 6 hours, RT. b Isolated yields relative to alkene are reported. c GC-MS yields using n-dodecane as an internal standard are reported in parentheses. d Alkene (0.5 mmol), dimethylmethoxysilane (2.5 mmol.), 24 hours. e Alkene (0.5 mmol), 2a (2.0 mmol).

Next, the reactions of alkenes with Me(EtO)2SiH and (MeO)3SiH were investigated. To our delight, the reactions gave various functionalized hydrosilanes in good yields (5a-6f, Scheme 3). An increase of the loading of the catalyst to 5 mol % was necessary. For the reactions of alkenes with Me(EtO)2SiH, (Alkyl)2(Me)SiH were formed, in which the two alkyl groups came from the alkene. Interestingly, for the reactions of alkenes with (MeO)3SiH, (Alkyl)2SiH2 were obtained as the major product, and only less than 3% of (Alkyl)3SiH was detected by GC-MS. Even when the alkene was in large excess (6 equiv.) and the reaction time was increased to 36 hours, the yield of (Alkyl)3SiH did not exceed 6%. The reaction of isolated dioctadecylsilane (6a) with 1.2 equivalent of 1-octadecene under the conditions of Scheme 3 was then monitored by 1H NMR. (n-octadecyl)2SiH2 (6a) was fully consumed in 1.5 hour, and  n-octadecyl)3SiH was produced in a nearly quantitative yield. On the other hand, if 3 equivalents of (MeO)4Si were added to the reaction mixture, the conversion of 6a was less than 40% in 6 hours. This result suggests that the (MeO)4Si byproduct inhibits the hydrosilylation of alkenes 6a-6f with (alkyl)2SiH2, leading to selective formation of dialkylsilanes in the reaction of alkenes with (MeO)3SiH.

Scheme 3. Ni-catalyzed synthesis of alkyl hydrosilanes using 2b-2c and alkenes a

a Typical conditions : for reactions with 2b, alkene (1.2 mmol), methyldiethoxysilane (1.5 mmol), 1a (5 mol %), NaOtBu (10 mol %),THF (4 mL), 24 hours, RT; for reaction with 2c, alkene (1.5 mmol), trimethoxysilane (2 mmol), 1a (5 mol %), NaOtBu (10 mol %),THF (4 mL), 24 hours, RT. b Isolated yields relative to intermediate silanes are reported. The theoretical yield of 5a-6f is 0.5 mmol. c Alkene (1.35 mmol) was used. d Alkene (1.8 mmol) was used. e Alkene (1.2 mmol) was used.

The above reactions could be applied for the synthesis of alkyl hydrosilanes containing different alkyl groups. For example (2-(cyclohex-3-en-1-yl)ethyl)(methyl)(octyl)silane (7a) was obtained in a 68% yield by step-wise addition of 1-octene and 4-vinylcyclohex-1-ene (eq. 5).

Several experiments were carried out to provide preliminary mechanistic insights in these reactions. The addition of a catalytic amount of NaOtBu to Me2(MeO)SiH led to a quick release of Me2SiH2, detected by 1H NMR. Likewise, the addition of a catalytic amount of NaOtBu to (MeO)3SiH led to the formation of SiH4, again detected by 1H NMR. Thus, the alkoxysilanes were proposed to react first with a base to give hydrosilanes in a sequence shown in eqs 6-7, Scheme 4. Nucleophilic attack of an alkoxide anion on Me2(MeO)SiH gave the 5-coordinate species (A), which reacted with another molecule of Me2(MeO)SiH to produce Me2SiH2 and Me2(MeO)2Si, in a disproportionation manner. The base was also regenerated in this step. Similar base-catalyzed disproportionation reactions of alkoxysilanes were previously reported. 9 The disproportionation of alkoxysilanes was followed by 1H NMR. Me2(MeO)SiH was converted to Me2SiH2 and Me2(MeO)2Si under 0.5 mol % of NaOtBu in THF-d8 in 50 min. Analogous reaction of Me(EtO)2SiH took 20 minutes to complete. The dispropotionation of trialkoxysilane (MeO)3SiH was even faster: in the presence of 0.1 mol % of NaOtBu the conversion was completed after 5 minutes. Thus, the rate of disproportionation follows the order: (MeO)3SiH > Me(EtO)2SiH  > Me2(MeO)SiH. It was noted that the nickel complex did not accelerate the rate of disprpoportionation (See SI).

Complex 1a was found to be an efficient catalyst for the hydrosilylation of 1-octene by Ph2SiH2 and Et2SiH2 in the presence of 1 equivalent of NaOtBu (relative to 1a), giving the respective hydrosilanes in good yields (eq. 8, Scheme 4). Accordingly, we propose that the present reactions proceed first by base-catalyzed disproportionation of alkoxysilanes to give hydrosilanes, which upon catalysis by 1a, hydrosilylated alkenes to give the alkylhydrosilanes.

Scheme 4. Proposed mechanism

Because Me2SiH2, MeSiH3 and SiH4 were proposed as intermediates in the hydrosilylation reactions reported here, it is important to compare the rate of their generation and that of hydrosilylation. For large scale applications, the rate of hydrosilylation should be faster than the rate of disproportionation in order to avoid the build-up of flammable intermediates, especially the pyrophoric SiH4. When an actual reaction of 1-decene with Me2(MeO)SiH was monitored by 1H NMR, a sub-stoichiometric amount of Me2SiH2 was detected in the beginning, but it disappeared after 15 minutes. This result suggests that the generation of Me2SiH2 is faster than hydrosilylation, but hydrosilylation is sufficiently fast so that the intermediate is quickly consumed. In the reaction of 1-decene with Me(EtO)2SiH, only Me(decyl)SiH2, but not MeSiH3, was detected as intermediate in the beginning of the reaction. This result suggests that hydrosilylation is faster than the generation of MeSiH3. And in an actual reaction of 1-decene with (MeO)3SiH, no SiH4 was detected by 1H NMR. The above results suggest that the reactions of alkenes with Me(EtO)2SiH or (MeO)3SiH described here are potentially suitable for large scale applications because there was no significant build-up of flammable MeSiH3 or pyrophoric SiH4.1 0 Even the reactions of alkenes with Me2(MeO)SiH might be suitable for large scale application given proper handling,  because only a small amount of Me2SiH2 was formed as intermediate in the beginning of the reaction. Me2SiH2 has good solubility in organic solvents and is not pyrophoric.

The superior activity of 1a compared to other nickel complexes (Table 1), is tentatively attributed to the former’s inability to catalyze isomerization of alkenes under basic conditions, as well as its low efficiency for the hydrosilylation with alkoxysilanes. It was found that under the reaction conditions of Table 1, various amounts of internal alkenes were formed using 1b-1d and NiCl2(PPh3)2 as catalysts. Only minor amount of internal decenes were formed during the reaction (<5% by GCMS) when 1a was used as the catalyst. Moreover, less than 3% of conventional hydrosilylation product (Decyl)(Me)2SiOMe was formed in the reaction of 1-decene with Me2(MeO)SiH using 1a as precatalyst, while up to 19% of this compound was formed using other nickel catalysts in Table 1. Likewise, for the reactions with 2b and 2c (Scheme 3) using 1a as precatalyst, only 3-8% of conventional hydrosilylation products were formed.

In summary, we have developed a novel catalytic method allowing the use of alkoxy hydrosilanes as surrogates of gaseous silanes in hydrosilylation reactions of alkenes. While serving as a convenient and safer alternative to methods using directly Me2SiH2, MeSiH3 and SiH4, the present approach actually facilitates the “formal” use of these underexplored reagents in chemical synthesis.

ASSOCIATED CONTENT

Supporting Information

The Supporting Information is available free of charge on the ACS Publications website.

Experimental procedures and 1H and 13C NMR spectra (PDF)

AUTHOR INFORMATION

Corresponding Author

* xile.hu@epfl.ch

Notes
The authors declare no competing financial interests.

ACKNOWLEDGMENT

This work is supported by the EPFL and the Swiss National Science Foundation (200020_162362).

REFERENCES

(1)	a) Griffiths, S. T.; Wilson, R. R. Combus. Flame, 1958, 2, 244; b) Chen, J.-R.; Tsai, H.-Y.; Chen, S.-K.; Pan, H.-R.; Hu, S.-C.; Shen, C.-C.; Kuan, C.-M.; Lee, Y.-C.; Wu, C.-C. Process Saf. Prog. 2006, 25, 237.

(2)	a) Salimgareeva, I. M.; Kaverin, V. V.; Jurjev, V. P. J. Organomet. Chem. 1978, 148, 23; b) Itoh, M.; Iwata, K.; Takeuchi, R.; Kobayashi, M. J. Organomet. Chem. 1991, 420, C5; c) Kobayashi, M.; Itoh, M. Chem. Lett. 1996, 25, 1013; d) Itoh, M.; Iwata, K.; Kobayashi, M. J. Organomet. Chem. 1999, 574, 241.

(3)	Selected reviews: a) Marciniec, B. Hydrosilylation: A Comprehensive Review on Recent Advances, Springer, Berlin, 2009; b). Marciniec, B. Coord. Chem. Rev. 2005, 249, 2374; c) Roy, A. K. Adv. Organomet. Chem. 2008, 55, 1; d) Putzien, S.; Nuyken, O.; Kühn, F. E. Prog. Polym. Sci. 2010, 35, 687; e) Nakajima, Y.; Shimada, S. RSC Adv. 2015, 5, 20603; Selected recent examples: f) Tondreau, A. M.; Atienza, C. C. H.; Weller, K. J.; Nye, S. A.; Lewis, K. M.; Delis, J. G. P.; Chirik, P. J. Science 2012, 335, 567; g) Peng, D.; Zhang, Y.; Du, X.; Zhang, L.; Leng, X.; Walter, M. D.; Huang, Z. J. Am. Chem. Soc. 2013, 135, 19154; h) Greenhalgh, M. D.; Frank, D. J.; Thomas, S. P. Adv. Synth. Catal. 2014, 356, 584; i) Noda, D.; Tahara, A.; Sunada, Y.; Nagashima, H. J. Am. Chem. Soc. 2016, 138, 2480-2483.

(4)	 a) Simonneau, A.; Oestreich, M. Angew. Chem. Int. Ed. 2013, 52, 11905; b) Simonneau, A.; Oestreich, M. Nat. Chem. 2015, 7, 816.

(5)	Buslov, I.; Becouse, J.; Mazza, S.; Montandon-Clerc, M.; Hu, X. Angew. Chem. Int. Ed. 2015, 127, 14731.

(6)	Caution: As shown in the later part of the paper, a small amount of Me2SiH2 was formed as an intermediate in the beginning of the reaction, but it was quickly consumed during the reaction course. Me2SiH2 is a flamable gas, so safety precaution should be made when opening the reaction vessel at the end of the reaction. No accident was encountered in our studies.

(7)	a) Kira, M.; Sato, K.; Sakurai, H. J. Org. Chem. 1987, 52, 948; b) Becker, B.; Corriu, R. J. P.; Guérin, C.; Henner, B.; Wang, Q. J. Organomet. Chem. 1989, 368, C25; c) Becker, B.; Corriu, R. J. P.; Guérin, C.; Henner, B.; Wang, Q. J. Organomet. Chem.1989, 359, C33;d) Chuit, C.; Corriu, R. J. P.; Reye, C.; Young, J. C. Chem. Rev. 1993, 93, 1371.

(8)	See Supporting Information

(9)	a) Friedel, C.; Ladenburg, A. Ann. Chim. Phys. 1871, 23, 430; b) Bailey, D. L. Patent CA 538521, 1957; c) Suzuki, E.; Nomoto, Y.; Ono, Y. Catal. Lett. 1997, 43, 155;

(10)	Caution: Even though we used this method without any incident and there was no build-up of MeSiH3 and SiH4 in the reactions, precautions are required to run these reactions in large scales due to the flammable nature of these intermediates.